1. Powertrain & Drivetrain

Engine: Cummins Z14E560 52 (EEV-certified), Euro V emissions, hydraulic retarder, 100,000 km oil change interval.

- Transmission: ZF AMT (12TX2621TD) with hydraulic retarder and Power Take-Off (PTO).

- Axles:

- Front: Sanhuan 7T disc brake axle (maintenance-free wheel ends).

- Rear: Hande 13T469 axle (disc brakes, differential lock, 100,000 km oil change interval).

2. Chassis & Suspension

-Frame: 710L steel, 270mm height, 8mm plate thickness.

- Wheelbase: 3800mm.

- Suspension:

-Front: Progressive leaf springs (4 leaves).

-Rear: Four-airbag suspension with ECAS (Electronically Controlled Air Suspension).

- Tires: Sailun 315/80R22.5, aluminum rims, front stripe/rear block tread, TPMS.

- Fuel Tank: Aluminum, >1200L capacity, water-heated.

3. Cab & Interior

- Structure: S23H high-roof double sleeper cab, flat-floor design, 2.5m width.

- Comfort Features:

- Adjustable leather seats (driver/passenger) with ventilation, heating, lumbar support, and multi-directional adjustments.

- Ambient lighting, wireless charging, 12.3-inch HD dashboard, and central screen.

- Parking AC/heater, onboard refrigerator, 1000W inverter.

- Storage: Dual side storage boxes, central storage platform (drawers, cup holders), sleeper compartments with refrigerator.

- Controls: Column shifter, EPB (Electronic Parking Brake), one-key start, multifunction steering wheel (volume, cruise control, Bluetooth).

4. Safety & Driver Assistance

- Active Safety:

- AEBS (Autonomous Emergency Braking), LDWS (Lane Departure Warning), FCWS (Forward Collision Warning), ESC (Electronic Stability Control), TPMS (Tire Pressure Monitoring), hill assist.

- Passive Safety:

- Airbag seats, three-point seatbelts, retro-reflective door warnings, and a fire extinguisher.

- Visibility & Monitoring:

- Four-way imaging system, heated/electric mirrors, night vision camera, automatic headlights/wipers.

5. Electrical & Lighting

- Exterior Lighting:

- LED headlights (auto-adjusting), fog lights, clearance lights, side turn signals, step lights, and cab rear light.

- Interior Lighting:

- Adjustable dome lights, sleeper compartment lights, and ambient lighting (strips and spotlights).

- Power Supply:

- 220Ah*2 battery, 220V EU outlet, 24V/12V/USB ports.

6. Customization & Compliance

- Branding: No logos or decals (Dongfeng, National VI, torque markings, etc.), and the front panel was redesigned.

- Regional Adaptations:

- English labels, Iranian radio bands, and Chinese-standard license plate mounting.

- Steering/reversing beep alerts (Iran compliance).

- Certifications: E-mark certified components, Euro V emissions.

7. Body & Exterior

- Aerodynamics: Adjustable top air deflector, flip-left side deflector.

- Utility Features:

- Jost 50# saddle (1180mm height), trailer ABS/power lines (7-pin/15-pin EU standard).

- Side toolboxes (metal + gas spring), driver-side dust blower.

- Material**: Aluminum fuel tank, laminated/tempered glass, plastic-stem mirrors.

8. Miscellaneous

- Soundproofing: Insulation + sound-absorbing cotton.

- Exclusions: No sunroof, T-BOX, driving recorder, air horn, or rear window/VASOL logo.

- Special Requests: Retained harness interface for driving recorder, ADR controller.

Key Highlights

- Compliance: Euro V, E-mark, Iranian radio/English labels.

- Driver-Centric: Ventilated/heated seats, dual sleeper, ambient lighting, ergonomic controls.

- Safety AEBS/LDWS/FCWS/ESC, four-way imaging, TPMS.

- Durability: Maintenance-free axles, 710L steel frame, corrosion-resistant components.